# Signal Processing & Coordinate Systems TRD

Status  Implementation specification / proposed design     Version  1.0  |  24 October 2026

Time correctness, coordinate geometry, filtering, interpolation and quality propagation.

Normative terms: MUST = required; SHOULD = preferred; MAY = optional. All numeric thresholds in these documents are provisional engineering targets until validated on target hardware and expert-annotated data.

# Coordinate and unit registry

| System | Units | Meaning |
| --- | --- | --- |
| image_normalized | [0,1] x,y | origin top-left, view dependent |
| image_pixel | px | raw sensor-aligned frame |
| estimated_world | meters if model specifies | model-estimated 3D, NOT calibrated capture |
| calibrated_world | meters | physical frame only with documented calibration |
| body_relative | normalized body units | root/scale and orientation specified |

Every feature MUST declare source coordinate system, handedness transform, origin, axis convention, units and view validity. Image-mirroring is metadata; reflect coordinates only once. Preserve raw and transformed landmarks.

# Time series handling

Use capture monotonic timestamps; never calculate velocities from nominal FPS when frames are dropped. Detect timestamp regression, duplicate samples and gaps. A gap > configurable phase threshold invalidates continuity and emits TIME_GAP.

Support time-indexed history windows (e.g. latest 2 seconds) with masks for missing joints and resampling events; do not interpolate across large motion gaps. Preserve source confidence and derived-data lineage.

# Filtering

Baseline: per-joint One-Euro filters with velocity-adaptive smoothing parameters tuned on held-out movement recordings. Initial parameter defaults MUST be treated as candidates, not validated constants.

Do not smooth across phase boundaries without checking induced delay. Offline analysis MAY use bidirectional filtering but must be marked retrospective and never used to claim live latency.

Joint outlier gate: detect physically implausible jumps relative to confidence and timestamp, mark observation invalid and optionally short-gap predict; predicted samples reduce evidence reliability.

# Data flow and invariants

RAW POSE -> COORDINATE VALIDATION -> MIRROR/ORIENTATION ->

ROOT+SCALE NORMALIZATION -> OUTLIER DETECTOR ->

CAUSAL FILTER -> TIME WINDOW -> FEATURES + QUALITY MASK

Validation invariants: finite coordinates; valid joint indexes; no division by zero; missing height/root produces normalized-invalid; derived vector retains source frame ids; no unwarranted absolute metric length claims.

# Contract and error semantics

All messages MUST include schema_version, session_id, event_id or frame_id, monotonic timestamp, model/rule provenance, and quality flags as appropriate. Never assume wall-clock time is monotonic.

Missing or occluded measurement MUST be represented by a nullable value and a typed reason, not zero and not an invented measurement. Coordinate frames and units are mandatory.

Recoverable failures use structured error codes; a dependent feature becomes NOT_ASSESSABLE without stopping camera capture. Unknown model IDs and incompatible contract versions are rejected closed.

# Observability and acceptance

Emit per-stage processing duration, queue age, discarded frame count, memory, dropped inference count, timeout and abstention reason. Do not log raw images or identifying media by default.

Unit tests: synthetic geometric fixtures and edge cases. Integration tests: golden pose sequences. Regression tests: representative sport packages. Benchmark sustained 20-minute sessions on reference devices.

Definition of done: schema validated, deterministic tests green, a specific failure fallback implemented, performance characterized on reference hardware, and documentation and telemetry updated.